SECTION 32 9300

PLANTS

*************************************************************************************************************

LANL MASTER SPECIFICATION

Word file at http://engstandards.lanl.gov

This template must be edited for each project.  In doing so, specifier must add job-specific requirements.  Brackets are used in the text to indicate designer choices or locations where text must be supplied by the designer.  Once the choice is made or text supplied, remove the brackets.  The specifications must also be edited to delete specification requirements for processes, items, or designs that are not included in the project -- and specifier’s notes such as these.  To seek a variance from requirements in the specifications that are applicable, contact the Engineering Standards Manual Civil and Architectural POCs. Please contact POC with suggestions for improvement as well.

When assembling a specification package, include applicable specifications from all Divisions, especially Division 1, General requirements.

Specification developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see ESM Chapter 1 Section Z10 Specifications and Quality sections.

This specification is to be used for the establishment or reestablishment of vegetation.  See ESM Civil Chapter 3, Section G2050, Landscaping and ESM Architectural Chapter 4, Section B-C_GEN, G2050 for additional guidance.

*************************************************************************************************************

- GENERAL

- SECTION INCLUDES

- Preparation of planting mix

- New trees, plants, and ground cover

- Mulch and fertilizer

- Maintenance

- Tree pruning

- RELATED SECTIONS

- Section 32 8400, Planting Irrigation

- ACTION SUBMITTALS

- Submit the following:

      - Planting schedule dates and type of work to be performed.  Correlate to provide specified maintenance periods.  Once schedule is accepted, document reasons for delays or changes and revise dates only as approved by the LANL Subcontract Technical Representative (STR).

      - Materials list of trees, ground cover, and shrubs.  Indicate source name, address, telephone number, quantity, genus, variety, size, and species of trees to be ordered and provide Installation Instructions for trees and shrubs.

      - Catalog data, including sources of supply for amendments, mulch, trees, ground cover, and shrubs.

- QUALITY ASSURANCE

- Subcontractor Qualifications

- Perform work with a licensed subcontractor experienced in landscaping with the type and scale of work required and having equipment and personnel adequate to perform the work satisfactorily.

- Material Quality Controls:

      - Ship landscape materials with certificates of inspection required by industry and/or government regulations.  Package standard products with manufacturer’s certified analysis.  Comply with regulations applicable to landscape materials.

      - Label each tree, shrub and ground cover with securely attached waterproof tag bearing legible designation of botanical and common name.

      - LANL STR shall inspect or tag trees, shrubs and ground covers either at place of growth or at site before planting, for compliance with requirements for genus, species, variety, size and quality, injuries and defects.  Unsatisfactory or defective material may be rejected at any time during progress of work.

- DELIVERY, STORAGE AND HANDLING

- Delivery/Storage of Materials:

      - Deliver packaged materials in containers showing weight, analysis and name of manufacturer.  Protect materials from deterioration during delivery and while stored at site.

      - Do not prune trees and shrubs prior to delivery.  Do not bend or bind-tie trees or shrubs in such manner as to damage bark, break branches or destroy natural shape.  Provide protective covering for deciduous tress or shrubs during transport to site.  Do not remove boxed, balled/burlaped, or container-grown stock from containers until planting time.

      - Delivery trees and shrubs after preparations for planting have been completed and can be planted immediately.  If planting is delayed set trees and shrubs in shade, protect from weather and mechanical damage and keep roots moist.

- Handling During Planting:

      - Lift trees only by methods that will not damage bark or loosen trunk from rootball or otherwise damage the appearance, health or growth of the tree.

      - Handle shrubs and ground covers by containers, do not carry by foliage or branches.  Do not drop or damage material during moving.

- ENVIRONMENTAL REQUIREMENTS

- Do not install plant life when ambient temperatures may drop below 35 degrees F or rise above 90 degrees F.

- Proceed with and complete landscape work as rapidly as portions of site become available, working within seasonal limitations for each kind of landscape work required.  Assess if site is ready for work to begin.

- Determine the acceptability of subgrade preparation prior to the start of landscaping work.  Protect existing and new utilities and construction, items to remain and ensure proper timing of each phase of work.

- Do not plant when conditions detrimental to plant growth are encountered, such as rubble fill, adverse drainage conditions, or obstructions.

- Coordinate plant locations with the location of irrigation heads.  When a discrepancy is found between the irrigation system layout and the planting layout, notify LANL STR immediately and do not proceed with further work in that area until notified by LANL STR.

- Plant trees and shrubs after final grades are established and prior to planting of lawns.  Protect existing lawn areas and promptly repair damage to lawns resulting from planting operations.

- WARRANTY

- Warrant trees, shrubs and ground covers for a period of one full growing season after date of final acceptance against defects including death and unsatisfactory growth.  Replace with plants of same size and species as specified, planted in the next growing season.

- Remove and replace trees, shrubs or other plants found to be dead or in unhealthy condition during warranty period.

- Another inspection will be conducted at end of extended warranty period, if any, to determine acceptance or rejection.  Only one replacement will be required at end of warranty period, except for losses or replacements due to failure to comply with specified requirements.

- Release of the site before final acceptance may be negotiated through LANL prior to termination of the subcontract.

PART 2	PRODUCTS

- PLANTING BACKFILL MIXTURE

- Provide a planting soil/backfill mixture to include a blend of topsoil, biocomp, compost, or peat moss installed in prepared tree and shrub pit excavations. “Planter’s mix” topsoil is preferred.

- SOIL AMENDMENTS

- Spread lawn starter fertilizer at a rate of 1 pound of nitrogen (N) per 1,000 sq. ft. or 1.2 to 1.5 pounds of phosphorus (P205) per 1000 sq. ft. after placing sod.

- Water: Clean, fresh, and free of substances or matter that could inhibit vigorous growth of plants.

- PLANT MATERIALS

- Provide sound, healthy symmetrical specimens typical of their species, with well-formed tops and good healthy root systems.  Provide materials free from pests, diseases, broken branches and plant sizes at least equal to the minimum size indicated.

- Nomenclature: Conform to Standardized Plant Names and accepted botanical nomenclature in the nursery trade.

- Strictly adhere to the American Standards for Nursery stock unless specific approval is given for particular plant material that does not meet those specifications.

- To ensure whole sound condition of the rootball, fully wrap the balled and burlapped material with no deterioration of the wrapping fabric.  Properly root prune balled and burlapped stock when dug and during cultivation.

- TREES AND SHRUBS

- Provide trees and shrubs, deciduous and evergreen, well shaped, symmetrical, full, sound, healthy specimens, with well-formed tops and healthy root systems.  Provide trees and shrubs that are free from injurious pests, diseases, broken branches, etc.

- Provide nursery grown evergreen trees that have been fully transplanted at least twice during cultivation.

- The rootball of balled and burlapped material shall be fully burlap wrapped with little deterioration of the fabric and the rootball shall be in whole sound condition.  Rootballs wrapped with plastic burlap or plastic twine will be rejected. Use rootball supporting devices, such as wire baskets to hold the ball in firm rigid condition.  Do not use trees that have had the main leader pruned and no replacement leader trained.  Trees which have trunk abrasions or scars comprising one-third or more of the diameter of the trunk or which diminish the aesthetic quality of the tree may be rejected.

- LANDSCAPE MATERIALS

  - Anti-Desiccant: Emulsion type, film-forming agent designed to permit transpiration but retard excessive loss of moisture from plants.  Deliver in manufacturer’s fully identified containers and mix in accordance with manufacturer’s instructions.

  - Landscape Fabric: Non-woven, spun bonded, water permeable polypropylene or polyester material with UV inhibitors in accordance with the following:

  - Tensile strength per ASTM D4632: minimum 135 lbs. min.  Elongation percent per ASTM D4632: >60%.  Equivalent opening size (US Sieve NO.): 60/70.  Weight in pounds per square yard: 3.0 oz/sq. yd. min.  Puncture strength (lbs) 35.  Do not expose fabric material to sunlight in storage

  - Staking Materials

- Stakes: Use green steel T-bar fence posts for tree support.

- Cable, Wire, Eye Bolts and Turnbuckles: Non-corrosive, of sufficient strength to withstand wind pressure and resulting movement of plant life.

- Fabric Pins: Steel pin minimum 11 gauge, 6”x1”x6”, per fabric manufacturer’s standard.

- Plant protectors: Rubber sleeves over cable to protect plant stems, trunks, and branches.

- Large Gravel Cover: 3/4 inch, coarse washed, natural color gravel, standard color shall be Santa Fe Brown.

PART 3	EXECUTION

- PREPARATION

  - Layout individual trees and shrubs and areas for multiple plantings.  Stake locations and outline areas and secure LANL acceptance before excavation for planting work.

  - Preparation of Planting Soil/Backfill Mixture:

- 1.	Before mixing, clean topsoil of roots, plants, clods, stones, clay lumps, and other extraneous materials harmful or toxic to plant growth.

- 2.	Mix specified soil amendments and fertilizers with topsoil at rates specified.  Delay mixing of fertilizer if planting will not follow within 2 days.

- PLANTING

  - Trees

- Excavate tree pits to a diameter 12 inches greater than the spread of the rootball, and 12 inches deeper than the depth of the rootball.

- Loosen a minimum depth of 3 inches of subsoil in bottom of pit and mix planting soil/backfill mixture into the loosened subsoil.  Place and compact soil/backfill mixture in pit to proper depth so that top of rootball will be 4 inches below surrounding finish grades.  On slopes, maintain 4 inches below surrounding grade on the downhill side.

- Set rootball on compacted soil/backfill mixture, plumb tree in center of pit with top of ball 4 inches lower than surrounding grade.  Soil/backfill mixture in carefully tamped layers to two-thirds the depth of the rootball and flood with water.  After water is absorbed continue backfilling and tamping to grade, leaving no voids or air pockets.  Water again after placing final layer of soil/backfill mixture.

- Around trees not in planter beds or lawns, form a 4 foot diameter depression 4 inches below surrounding grade with sides neatly sloped inward to top of rootball.  On slopes, maintain 4 inches below surrounding grade on the downhill side.   Fill depressions with landscape fabric and gravel cover.

- If specified on Drawings, guy or stake tree.

  - Shrub Areas:

- Final grading shall be accomplished and shrub locations will be approved by LANL prior to shrub pit excavation.

- Clear shrub areas of rocks, stones larger than 1 inch across and other debris.

- Place individual shrubs in pits a minimum of 6 inches deeper and 12 inches wider in diameter than the container or ball size.  Place plants on compacted soil/backfill layer.  When shrub pit is two-thirds backfilled.  Place soil/backfill mixture to finish grade level and thoroughly soak.

- Set top of soil/backfill mixture in shrub areas to allow for large rock cover

- Install landscape fabric and large rock cover around shrub as required on Drawings.

- When not in planting beds, install individual shrubs or small shrub groups with a watering depression a minimum of 3 inches deeper than adjacent grades on downhill side of depression and 18 inches wider than the rootball of the shrub or the combined area of the shrub group.  Coordinate shrub placement with the location of bubbler or drip emitter.

  - Ground Cover and Multiple Plant Areas:

    - Loosen subgrade of planting bed areas to a minimum depth of 6 inches.  Remove stones larger than 1 inch in any dimension, sticks, stones, rubbish and other extraneous matter.

    - Place approximately 1/2 of total amount of planting soil required.  Work into top of loosened subgrade to create a transition layer, then place remainder of the planting soil.  Ensure top of planting soil mixture is smooth, level and consistent.  Install planting soil mixture to minimum depth required to meet lines, grades and elevations required, after light rolling, natural settlement and depth of mulch.

    - After soil preparation, set out ground cover materials at on center rates specified.  Align plants in uniform rectangular pattern or triangular pattern.  Excavate pits large enough to set each plant.  Backfill with excavated planting soil mix.

    - Water area thoroughly after planting. Fill depressions and level high spots.

  - Planters With Covered Bottom: Exterior Planters

      - Place a minimum 4 inch layer of 3/4 inch gravel in bottom of planters.  Install landscape fabric to cover gravel, lap edges by 6 inches at joints and up side of planter.  Fill with soil mixture consisting of 1 part topsoil, 1 part course sand, and 1 part peat humus.  Place soil in lightly compacted layers to an elevation of 1-1/2 inch below top of planter, allowing for natural settlement and 2 inch depth of mulch.

      - Provide for irrigation to the planters with the installation of a bubbler or drip emitter system.

  - Mulch: Provide not less than the following thicknesses of mulch and finish level with adjacent finish grades.

      - Install landscape fabric per manufacturer’s instructions; to include overlapping edges and pinning.  Neatly trim edges and areas of abutment without having ragged edges.  Fabric shall be laid onto abutments; sidewalks, walls, landscape edging.  Do not leave gapes at abutment areas.

      - Provide a minimum of 3 inch thickness of mulch (stone) at indicated hard scaped areas, unless otherwise noted on Drawings.

      - Provide a minimum 2 inch thickness of mulch at ground cover and shrub planting bed, unless otherwise noted on Drawings.

      - Provide a minimum 3 inch thickness of mulch at tree pits, unless otherwise noted on Drawings.

      - Install mulch adjacent to walks and curbs abutting the walk or curb edge with top of mulch one inch below top of walk or curb.

  - Prune, thin out and shape trees and shrubs for structure, appearance, and health.  Prune trees to retain required height and spread.  Do not cut tree leaders, and remove only injured or dead branches from flowering trees.  Prune shrubs to retain natural character and accomplish their function in the landscape design.

      - Remove and replace excessively pruned or malformed stock resulting from improper pruning.

- MAINTENANCE

- Begin maintenance immediately after planting.

- Maintain trees, shrubs and other plants until final acceptance.

- CLEANUP AND PROTECTION

  - During landscape work, keep pavements clean and work area in an orderly condition.

  - Protect landscape work and materials from damage due to landscape operations, operations by other subcontractors, trades and trespassers.  Maintain protection during installation and maintenance periods.  Treat, repair or replace damaged landscape.

  - Upon completion of all planting operations, clean the portion of the project site used for storing materials and equipment of all debris.  Remove all superfluous materials and equipment from the project site.  Sweep or wash walks and pavement clean upon completion of work in this section.

- INSPECTION AND ACCEPTANCE

- Upon request when landscaping work is completed including maintenance, LANL will inspect to determine acceptability of work.

- Where inspected landscape work does not comply with requirements, replace rejected work and continue specified maintenance until reinspected by LANL and found to be acceptable.  Remove rejected plants and materials promptly from project site.

END OF SECTION

*************************************************************************

Do not delete the following reference information:

*************************************************************************

FOR LANL USE ONLY

This project specification is based on LANL Master Specification 32 9300 Rev. 4, dated August 27, 2012.